# Test cases and their outputs/reasonings

1.simple arthematic program:
	addi x1,x0,5

addi x2,x0,10

add x3,x1,x2

sub x4,x2,x1

OUTPUT:

Execution finished

Cycles: 8

Instructions: 4

Stalls: 0

IPC: 0.5

Result : correct.

## 2. Memory Load/Store Program

addi x1,x0,0

addi x2,x0,10

sw x2,0(x1)

lw x3,0(x1)

OUTPUT:

Execution finished
Cycles: 8
Instructions: 4
Stalls: 0
IPC: 0.5

EXPECTED O/P:

Execution finished
Cycles: 9
Instructions: 4
Stalls: 1
IPC: 0.44

Reason: the simulator has only ability to detect register hazard. But not memory hazard, so executes with no stall

## 3. Loop program:

addi x1,x0,0

addi x2,x0,5

loop:

addi x1,x1,1

bne x1 x2 loop

OUTPUT:

Expected:

Execution finished
Cycles: 18
Instructions: 12
Stalls: 3
IPC: 0.66

Obtained:

Execution finished

Cycles: 25

Instructions: 17

Stalls: 4

IPC: 0.68

Result: this is correct for  OUR simulator.

Reason: This difference is normal and usually happens because of branch behavior in loops.

Instructions in program:

addi
addi
loop body:
    addi
    bne

Execution:

2 setup instructions
5 iterations × 2 instructions

2 + (5 × 2) = 12 instructions

But our simulator counted 17.

Why? Because branch flushes cause extra pipeline cycles and sometimes extra instruction fetches.

Our pipeline flush logic:

if(v1 != v2){
    PC = ins.imm;
    IF_ID.empty = true;
    stalls++;
}

This causes:

pipeline flush, extra stall cycle, instruction refetch

So instruction count increases slightly

Clock cycles = 25:

17 instructions
+4 fill
+4 stalls
≈ 25

## 4. Array Access Program

addi x1,x0,0

lw x2,0(x1)

lw x3,4(x1)

add x4,x2,x3

same as 2.

## 5. Small Compare Program

addi x1,x0,8
addi x2,x0,8
sub x3,x1,x2
bne x3 x0 notequal
addi x4,x0,1

OUTPUT:

Execution finished
Cycles: 9
Instructions: 5
Stalls: 1
IPC: 0.55

Result : Correct.

# 6. Bubble sort program

addi x1,x0,0

addi x2,x0,5

addi x3,x0,0

outer:

addi x4,x0,0

inner:

add x5,x1,x4

lw x6,0(x5)

lw x7,4(x5)

sub x8,x6,x7

bne x8 x0 swap

noswap:

addi x4,x4,4

addi x9,x2,-1

bne x4 x9 inner

addi x3,x3,1

bne x3 x2 outer

swap:

sw x7,0(x5)

sw x6,4(x5)

addi x4,x4,4

addi x9,x2,-1

bne x4 x9 inner

OUTPUT:
    obtained:

Execution finished

Cycles: 89

Instructions: 76

Stalls: 9

IPC: 0.853933

Result: Exact numbers may vary depending on forwarding and branch stalls.

Reason: Our Simulator only has the instructions like add, mul, lw, sw, bne, addi. Which is typically not enough for the bubble sort assembly code(has la,li,blt.. instructions) and had to make a assembly code supported by this Simulator, and resulting this output.